Roll Calculator

User Guide

Version: 2.0

Date: March 2026

Audience: Sales & Logistics

# 1. Overview

The Roll Calculator is an internal tool designed to determine the resulting roll diameter and weight for Naue geosynthetic products based on material dimensions and core specifications.

It is intended to support Sales in confirming product configurability and Logistics in planning transport and storage requirements.

# 2. Getting Started

Follow these steps to use the Roll Calculator:

- Open the Roll Calculator tool.

- Select the product category from the dropdown menu.

- Select the production site (Germany or Malaysia) – this affects available core options and maximum limits.

- Enter the required material dimensions (length, width, thickness).

- Optionally enter a thickness tolerance for worst-case diameter estimation.

- Optionally enter an article number to automatically pre-fill the product thickness from the internal database (273 articles available).

- Select the appropriate core diameter from the available options.

- Review the calculated outputs and any warnings displayed.

# 3. Article Number Lookup

The Roll Calculator includes a built-in article number lookup for all Naue products. By entering a valid article number, the product thickness is automatically filled in, reducing manual entry and the risk of input errors.

### How It Works

- An article number input field is displayed at the top of each calculation tab.

- As you type, a dropdown list of matching article numbers and names appears.

- Once a valid article number is entered, the product thickness (mean value) is automatically filled in the thickness field.

- The article name and thickness value are shown below the input field for confirmation.

- The field is optional – it can be left blank to enter the thickness manually.

### Visual Feedback

The article number field provides immediate visual feedback:

- ✅  Green border and article name displayed — article number found in database.

- ❌  Red border and error message — article number not found in database.

### Database

The lookup database contains 273 articles across all product families (Bentofix, Secutex, Carbofol, Secumat, Secudrain, Stex and others). Thickness values are based on mean measurement values from quality control data.

# 4. Input Parameters

| Parameter | Description | Notes |
| --- | --- | --- |
| Product Category | Select from the available product list | Determines applicable limits |
| Production Site | Germany (DE) or Malaysia (MY) | Affects max. roll diameter |
| Material Length | Total length of the roll [m] | Required |
| Material Width | Width of the roll [m] | Required |
| Material Thickness | Nominal thickness of the material [mm] | Required |
| Thickness Tolerance | Production tolerance for thickness [mm] | Optional; affects max. diameter |
| Article Number | Optional article number for automatic thickness lookup | Optional; auto-fills product thickness |
| Core Diameter | Select from available core options | See product-specific options below |

### Core Options by Product

| Product | Available Cores |
| --- | --- |
| Bentofix | 150 mm / 170 mm / 200 mm (steel core) |
| Secutex | 150 mm |
| Carbofol | 170 mm |
| Secumat | 150 mm |
| Secudrain | 150 mm |

Note: Steel cores are available with 150 mm and 200 mm outer diameter. For Bentofix rolls exceeding 1,750 kg, a 170 mm core is required.

# 5. Output Values

| Output | Description | Unit |
| --- | --- | --- |
| Roll Diameter | Calculated outer diameter of the finished roll | mm |
| Roll Weight | Estimated total weight of the roll | kg |
| Number of Windings | Calculated number of material layers on the roll | – |

The outputs are updated automatically as input parameters are changed. Any value approaching or exceeding a defined limit is highlighted accordingly.

# 6. Limit Values & Warnings

The calculator enforces product-specific limits for roll diameter and roll weight. If a calculated value exceeds a limit, a warning is displayed and the affected output field is visually highlighted.

### Warning Levels

| Status | Meaning |
| --- | --- |
| ✅ OK | All values are within the permitted limits. |
| ⚠️ Warning | Value is approaching the limit (within tolerance range). |
| ❌ Exceeded | Value exceeds the permitted maximum – configuration is not valid. |

Note: Configurations marked as Exceeded cannot be confirmed for production or dispatch without explicit approval.

# 7. Product-Specific Limits

### Bentofix

| Parameter | Germany (DE) | Malaysia (MY) |
| --- | --- | --- |
| Max. Roll Diameter | 890 mm | 1,000 mm |
| Max. Roll Weight | 2,500 kg | 2,500 kg |
| Core (< 1,750 kg) | 150 mm | 150 mm |
| Core (≥ 1,750 kg) | 170 mm (required) | 170 mm (required) |

### Secutex

| Parameter | Germany (DE) | Malaysia (MY) |
| --- | --- | --- |
| Max. Roll Diameter | 1,100 mm | 1,100 mm |
| Max. Roll Weight | 2,000 kg | 2,000 kg |
| Core | 150 mm | 150 mm |

### Carbofol

| Parameter | Germany (DE) | Malaysia (MY) |
| --- | --- | --- |
| Max. Roll Diameter | 1,100 mm | – (not applicable) |
| Max. Roll Weight | 2,500 kg | – (not applicable) |
| Core | 170 mm | – |

Note: Carbofol is produced in Germany only.

### Secumat

| Parameter | Germany (DE) | Malaysia (MY) |
| --- | --- | --- |
| Max. Roll Diameter | 800 mm | – (not applicable) |
| Max. Roll Weight | No limit | – (not applicable) |
| Core | 150 mm | – |

Note: Secumat is produced in Germany only.

### Secudrain

| Parameter | Germany (DE) | Malaysia (MY) |
| --- | --- | --- |
| Max. Roll Diameter | 800 mm | – (not applicable) |
| Max. Roll Weight | No limit | – (not applicable) |
| Core | 150 mm | – |

Note: Secudrain is produced in Germany only.

# Appendix – Formulas

### Roll Diameter

The outer roll diameter is calculated using the standard winding formula for cylindrical rolls:

D_outer = √( D_core² + (4 × L × t) / π )

| Symbol | Description | Unit |
| --- | --- | --- |
| D_outer | Outer roll diameter | mm |
| D_core | Outer diameter of the core | mm |
| L | Total material length | mm |
| t | Material thickness (nominal ± tolerance) | mm |

Note: When a thickness tolerance is entered, the calculator uses nominal thickness + tolerance for the worst-case (maximum) diameter estimate.

### Number of Windings

n = ( D_outer − D_core ) / ( 2 × t )

### Roll Weight

W = L × w × ρ

| Symbol | Description | Unit |
| --- | --- | --- |
| W | Roll weight | kg |
| L | Material length | m |
| w | Material width | m |
| ρ | Area weight (mass per m²) | kg/m² |

This document is for internal use only. For questions or corrections, please contact the responsible product manager.